🧪 Experiment:12 : Control LED using ESP32 and Blynk

🎯 Aim

To control an LED connected to ESP32 (GPIO 2) using the Blynk mobile application.

🧰 Requirements

- ESP32

- LED

- 220Ω resistor

- Mobile phone with Blynk app

- WiFi connection

⚙️ Procedure

Step 1: Wiring

- Connect LED positive terminal to GPIO 2 (D2)

- Connect LED negative terminal to GND through 220Ω resistor

Step 2: Install and Setup Blynk

- Install Blynk IoT app

- Create account and login

- Click New Template

  - Template Name: LED Control

  - Hardware: ESP32

  - Connection Type: WiFi

Step 3: Get Authentication Token

- Open template

- Go to Device Info

- Copy the Auth Token

Step 4: Create Datastream

- Create a new datastream

  - Name: LED

  - Virtual Pin: V0

  - Data Type: Integer

Step 5: Add Button Widget

- Choose Devices from dashboard and select “From Template”

Select “Create”

Now you can see a device is created and Api is created For your IoT device Copy this and use this on your code

- Then Select “Edit Dashboard”

- Drag “Switch” button to Dashboard

- Select “Settings”

- Select Led (V0) , which we configure already

- Finally select “Save And Apply ”

Step 6: Arduino Programming

- Open Arduino IDE

- Install required libraries:

  - Blynk

  - WiFi (ESP32)

ESP32 Configuration in Arduino IDE

✅ Step 1: Open Arduino IDE

- Open Arduino IDE on your laptop

✅ Step 2: Add ESP32 Board URL

- Go to File → Preferences

- Find “Additional Board Manager URLs”

- Paste this link:

https://dl.espressif.com/dl/package_esp32_index.json

- Click OK

✅ Step 3: Install ESP32 Boards

- Go to Tools → Board → Boards Manager

- Search: ESP32

- Install:
👉 “ESP32 by Espressif Systems”

✅ Step 4: Select ESP32 Board

Go to:

Tools → Board → ESP32 Arduino

Select:
👉 ESP32 Dev Module (most common)

✅ Step 5: Select COM Port

- Connect ESP32 via USB

- Go to:

Tools → Port

- Select correct COM port (e.g., COM3, COM4)

✅ Step 6: Install Blynk Library

- Go to Sketch → Include Library → Manage Libraries

- Search: Blynk

- Install:
👉 Blynk by Volodymyr Shymanskyy

Enter the following code:

#define BLYNK_TEMPLATE_ID "Enter you ID"

#define BLYNK_TEMPLATE_NAME "LED Control"

#define BLYNK_AUTH_TOKEN "Enter your Token"

#define BLYNK_PRINT Serial

#include <WiFi.h>

#include <BlynkSimpleEsp32.h>

// 🔹 WiFi credentials

char ssid[] = "Enter your WiFi name";

char pass[] = "Wifi Password";

int ledPin = 2;

// 🔘 Blynk button

BLYNK_WRITE(V0) {

digitalWrite(ledPin, param.asInt());

}

void setup() {

Serial.begin(115200);

pinMode(ledPin, OUTPUT);

Serial.println("Starting WiFi...");

WiFi.begin(ssid, pass);

// 🔥 Continuous connection status

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");   // prints dots continuously

}

Serial.println("\nWiFi Connected!");

Serial.print("IP Address: ");

Serial.println(WiFi.localIP());

// Connect to Blynk

Serial.println("Connecting to Blynk...");

Blynk.config(BLYNK_AUTH_TOKEN);

while (!Blynk.connected()) {

Blynk.connect();

Serial.print("*");   // shows Blynk connecting

delay(500);

}

Serial.println("\nBlynk Connected!");

}

void loop() {

Blynk.run();

Finally upload the code:

Output :